CN LAB # 2 | Muhammad Baqar Laghari | 24K-0006 | BAI-5A

QUESTION 1:

1.

DHCP maintains a lease pool/database of assigned IPs. Before offering an address, it checks this pool (and may send an ARP/ping check) to confirm the address isn't already leased. Each IP is leased to one MAC address at a time, and the server marks it "in use" until the lease expires or is released.

2. 
Use a Class C private range with a subnet mask that supports at least 50 hosts, e.g. 192.168.1.0/26 (255.255.255.192) giving 62 usable hosts. Reasons: private addressing avoids public IP cost/exposure, and /26 minimizes wasted addresses while leaving room for growth, better than a full /24 with 254 hosts.

3.

- Check physical connections/link lights.

- Verify IP address, subnet mask, and gateway on both devices (same subnet).

- Ping loopback (127.0.0.1), then own IP, then the other device.

- Check for IP conflicts.

- Inspect switch/hub port status and cable type (straight-through vs crossover).

- Check firewall settings on devices.

- Use tracert/ipconfig to isolate where communication fails.

4.

Exclude the statically assigned IP from the DHCP pool (add it to the exclusion list) so the server never offers it again. Additionally, release/renew the conflicting DHCP lease on the affected device or reassign it a different static/dynamic address.

5. 
A hub broadcasts data to all ports (shared bandwidth, half-duplex, collisions increase as devices grow, one collision domain). A switch forwards frames only to the destination port using MAC address tables (full-duplex, dedicated bandwidth per port, separate collision domains). Replacing hub with switch significantly improves speed, reduces collisions, and improves overall network efficiency.

6.

- DHCP Discover: client broadcasts a request for an IP.

- DHCP Offer: server responds offering an available IP from its pool.

- DHCP Request: client broadcasts acceptance of the offered IP.

- DHCP Acknowledgment: server confirms and finalizes the lease, and client configures its interface with the assigned IP, subnet mask, gateway, and DNS.

QUESTION 2:

Question 3: