Computer Networks Lab

CL

-

3001

|  Fall 2026  |  Section BAI

-

5

A

Course Instructor: Mr. Abdullah Shaikh

LAB # 01 — TASKS

Networking Commands, Topologies & IP Addressing

| Student Name |  | Roll No. / ID |  |
| --- | --- | --- | --- |
| Section |  | Date of Submission |  |

Submission Guidelines - Complete all tasks in this Word file and submit it as a PDF on Google Classroom before the deadline. - Name the PDF file as LabCourseName_Lab#-YourName-YourRollNo/ID — e.g. CN_Lab#01-Abdullah Shaikh-25K-7826. - No submission will be accepted after the deadline.

# TASK # 01  —  NETWORKING COMMANDS

Practice the following networking commands. Run each command in the terminal or command prompt, take a screenshot of the output, and write a one-line description of how it works. Paste the screenshots underneath the table, clearly labelled with the command name.

| S. No. | Command | One-line Description (of its working) |
| --- | --- | --- |
| 1 | ipconfig |  |
| 2 | ipconfig /all |  |
| 3 | ping |  |
| 4 | hostname |  |
| 5 | tracert |  |
| 6 | nslookup |  |
| 7 | netstat |  |
| 8 | route print |  |

Screenshots (label each one with the corresponding command):

PASTE SCREENSHOTS HERE

# TASK # 02  —  NETWORK TOPOLOGIES

Identify the given topologies and determine how many cables and ports are required to build such a network.

| Topology (a) | Topology (b) |
| --- | --- |
| Topology name: | Topology name: |
| Cables required: | Cables required: |
| Ports required: | Ports required: |

Working / justification:

SHOW YOUR CALCULATION

# TASK # 03  	NETWORK ID & BROADCAST ADDRESSES

Determine the Network ID, Limited Broadcast Address, and Directed Broadcast Address for the following IP addresses.

| IP Address | Network ID | Limited Broadcast Address | Directed Broadcast Address |
| --- | --- | --- | --- |
| 10.0.0.25 |  |  |  |
| 192.168.1.100 |  |  |  |
| 172.16.5.50 |  |  |  |
| 192.168.0.200 |  |  |  |
| 150.25.1.10 |  |  |  |
| 10.1.2.3 |  |  |  |
| 192.168.10.255 |  |  |  |
| 172.31.255.255 |  |  |  |
| 128.0.0.1 |  |  |  |
| 192.0.2.15 |  |  |  |

# TASK # 04  —  IP ADDRESS CLASSES

Identify the class (A, B, C, D, or E) of the following IP addresses.

| # | IP Address | Class | # | IP Address | Class |
| --- | --- | --- | --- | --- | --- |
| 1 | 192.168.1.1 |  | 11 | 248.0.0.5 |  |
| 2 | 10.0.0.1 |  | 12 | 11.0.0.50 |  |
| 3 | 224.0.0.5 |  | 13 | 192.0.2.10 |  |
| 4 | 172.16.5.10 |  | 14 | 224.5.5.5 |  |
| 5 | 240.0.0.1 |  | 15 | 128.50.10.25 |  |
| 6 | 128.1.0.10 |  | 16 | 10.10.10.1 |  |
| 7 | 192.168.0.100 |  | 17 | 172.20.1.100 |  |
| 8 | 50.0.0.25 |  | 18 | 192.168.5.5 |  |
| 9 | 172.31.255.254 |  | 19 | 240.5.5.5 |  |
| 10 | 230.1.1.1 |  | 20 | 1.1.1.1 |  |

# TASK # 05  	NETWORK BITS & HOST BITS

Identify the network bits and host bits for the following IP addresses. Highlight the network bits in yellow and the host bits in green.

| # | IP Address | Network Bits (yellow) / Host Bits (green) |
| --- | --- | --- |
| 1 | 10.0.0.1 |  |
| 2 | 192.168.1.1 |  |
| 3 | 128.16.5.200 |  |
| 4 | 50.25.50.100 |  |
| 5 | 172.31.255.255 |  |
| 6 | 198.51.100.10 |  |
| 7 | 193.0.0.100 |  |
| 8 | 100.75.200.25 |  |
| 9 | 150.200.100.50 |  |
| 10 | 192.168.10.25 |  |

# TASK # 07  —  NETWORK ID & HOST ID FROM SUBNET MASK

Given the following IP addresses and their subnet masks, calculate the Network ID and the Host ID.

| # | IP Address | Subnet Mask | Network ID | Host ID |
| --- | --- | --- | --- | --- |
| 1 | 10.0.0.1 | 255.0.0.0 | 10.0.0.0 | 0.0.0.1 |
| 2 | 172.16.5.10 | 255.255.0.0 | 172.16.0.0 | 0.0.5.10 |
| 3 | 192.168.1.100 | 255.255.255.0 | 192.168.1.0 | 0.0.0.100 |
| 4 | 128.10.20.30 | 255.255.0.0 | 128.10.0.0 | 0.0.20.30 |
| 5 | 11.0.0.25 | 255.0.0.0 | 11.0.0.0 | 0.0.0.25 |
| 6 | 150.25.50.100 | 255.255.0.0 | 150.25.0.0 | 0.0.50.100 |
| 7 | 192.168.10.255 | 255.255.255.0 | 192.168.10.0 | 0.0.0.255 |
| 8 | 50.100.0.1 | 255.0.0.0 | 50.0.0.0 | 0.100.0.1 |
| 9 | 172.31.255.255 | 255.255.0.0 | 172.31.0.0 | 0.0.255.255 |
| 10 | 100.100.100.100 | 255.0.0.0 | 100.0.0.0 | 0.100.100.100 |

MUHAMMAD BAQAR LAGHARI | 24K-0006 | BAI-5A | CN LAB # 1

# TASK # 08  	NUMBER SYSTEMS & IPV6 NOTATION

Do as directed.

a) Given the IP address 11000000.10101000.00000001.01011100 in binary:

- Convert it to decimal notation.

- Convert it to hexadecimal notation.

## ANSWER

b) Write the following IPv6 addresses in concise notation:

- 2001:0db8:0000:0000:0000:ff00:0042:8329

- 2001:0db8:85a3:0000:0000:8a2e:0370:7334

ANSWER

# TASK # 09  	VALID HOST RANGE

For the following IP addresses, calculate the second valid host ID and the last valid host ID.

| # | IP Address | Second Valid Host ID | Last Valid Host ID |
| --- | --- | --- | --- |
| 1 | 10.15.30.45 |  |  |
| 2 | 172.30.100.50 |  |  |
| 3 | 192.168.5.75 |  |  |
| 4 | 8.20.40.60 |  |  |
| 5 | 192.0.2.150 |  |  |

# TASK # 10  —  HANDS-ON NETWORK INVESTIGATION

Perform the following on your own machine and attach a labelled screenshot for each part.

- Find the IP address of your computer and identify the default gateway for your network connection.

- Test network connectivity to www.google.com and report the round-trip time along with the success or failure of the connection.

- Check and display the hostname of your computer. What name is assigned to your device on the local network?

- Trace the network path to a website such as www.wikipedia.org, and document how many routers the data passes through along with their respective IP addresses.

PASTE SCREENSHOTS AND OBSERVATIONS HERE

# TASK # 11  	TROUBLESHOOTING

Give a reason for the following:

a) A ping to 10.0.0.5 works, but a ping to the machine's name “server01” fails. What could be the reason for this issue?

## ANSWER

The End!

Good luck — remember to convert to PDF before submitting.