CN LAB 5	24K-0006	Muhammad Baqar Laghari

Task 1

Two LANs are connected through Router0. All end devices use static addressing. The FTP server is on the server-side LAN and both router interfaces are configured through the CLI.

Table 1 - IP Addressing

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Router0 | Fa0/0 | 10.6.0.1 | 255.255.0.0 | - |
| Router0 | Fa0/1 | 6.0.0.1 | 255.0.0.0 | - |
| PC0 | Fa0 | 10.6.0.2 | 255.255.0.0 | 10.6.0.1 |
| PC1 | Fa0 | 10.6.0.3 | 255.255.0.0 | 10.6.0.1 |
| PC2 | Fa0 | 10.6.0.4 | 255.255.0.0 | 10.6.0.1 |
| PC3 | Fa0 | 6.0.0.3 | 255.0.0.0 | 6.0.0.1 |
| FTP Server | Fa0 | 6.0.0.2 | 255.0.0.0 | 6.0.0.1 |

Router CLI Configuration

enable
configure terminal
hostname Router0
interface fastEthernet 0/0
 ip address 10.6.0.1 255.255.0.0
 no shutdown
 exit
interface fastEthernet 0/1
 ip address 6.0.0.1 255.0.0.0
 no shutdown
end
write memory
show ip interface brief
show running-config

FTP Server Setup

FTP was enabled on Server0. The user name is Muhammad and the password is 24K-0006. Write, Read, Delete, Rename and List permissions are enabled, and the default cisco account is removed.

Connectivity Test

PC0 and PC2 can reach the FTP server at 6.0.0.2. The following output shows a successful ping from the client LAN.

FTP Test - PC0 Upload

The file 24K-0006.txt contains: Muhammad Baqar Laghari, 24K-0006, BAI-5A. It is uploaded from PC0 using the FTP client.

FTP Test - PC3 Download and Delete

PC3 logs in from the other subnet, downloads the same file, deletes it from the server and lists the directory again.

Task 2

The mail server is connected directly to Router0 Fa0/1 with a copper cross-over cable. SMTP, POP3 and DNS are enabled on the same server. All clients use mail.baqar.com in their mail settings.

IP Addressing

| Device | Int. | IP Address | Subnet Mask | Gateway | DNS |
| --- | --- | --- | --- | --- | --- |
| Router0 | Fa0/0 | 10.6.0.1 | 255.255.0.0 | - | - |
| Router0 | Fa0/1 | 6.0.0.1 | 255.0.0.0 | - | - |
| PC0 | Fa0 | 10.6.0.2 | 255.255.0.0 | 10.6.0.1 | 6.0.0.2 |
| PC1 | Fa0 | 10.6.0.3 | 255.255.0.0 | 10.6.0.1 | 6.0.0.2 |
| PC2 | Fa0 | 10.6.0.4 | 255.255.0.0 | 10.6.0.1 | 6.0.0.2 |
| Mail Server | Fa0 | 6.0.0.2 | 255.0.0.0 | 6.0.0.1 | 6.0.0.2 |

Router CLI Verification

EMAIL and DNS Server Configuration

SMTP and POP3 are turned on and the domain is baqar.com. Three mail accounts are created for PC0, PC1 and PC2. DNS contains an A record that maps mail.baqar.com to 6.0.0.2.

Table 2 - Email Account Plan

| Device | Display Name | Email Address | Incoming / Outgoing Server | Password |
| --- | --- | --- | --- | --- |
| PC0 | Muhammad Baqar Laghari | pc0@baqar.com | mail.baqar.com | pc0pass |
| PC1 | Student PC1 | pc1@baqar.com | mail.baqar.com | pc1pass |
| PC2 | Student PC2 | pc2@baqar.com | mail.baqar.com | pc2pass |

Mail Client Configuration

The same incoming and outgoing server name is used on all three PCs. The DNS server field on every PC is 6.0.0.2.

Email Tests

PC0 sends the required message to PC1. PC1 receives it and replies successfully. A second message can be sent from PC1 to PC2 to verify the third account. An invalid recipient is also tested to confirm the error response.